Tender Text

Granule pot- and pan washing machine; Granule Flexi 
32A, Cold water feed

Short Product Description of Granule Pot- and Pan Washing Machine
· 3 programs with Granules and 3 programs with high-powered water

· Complying with Hygiene guidelines and norms DIN 10512 and NSF/ANSI 3.
· Capacity (per hour): 96 GN 1/1, up to 200 mm depth or 192 GN 1/2, up to 200 mm depth
· Capacity per program (pots & pans): 4 x GN 1/1, up to 200 mm depth or up 8 x GN 1/2 up to 200 mm depth
· Loading in two levels
· Operator’s panel placed at eye level
· Count-down of remaining time shown in the display during the program

· Led lights alerting when wash cycle is finished

· HACCP log accessed via a USB port or the Display
· Possible to add heat exchanger as an option
______________________________________________________________________________

Full Product Description of Granule Pot- and Pan Washing Machine

Frontloaded Granule pot- and pan washing machine. Flexible loading system in two levels based on Granule pot washing technology.
Efficient washing of GN (Gastronorm) containers, pots, bowls, saucepans, baking trays and utensils. Capacity per hour is 96 GN 1/1 up to 200 mm depth or up to 192 GN 1/2 up to 200 mm depth.
The powerful mechanical action of the Granule technology eliminates any need for soaking, scrubbing or pre-washing.

Food-safety is secured through a controlled amount of high temperature rinse water, fulfilling the requirements of DIN Norm 10512. The rinse with 8 l of fresh water of 85°C for 40 seconds, ensures a surface temperature of min 72 ºC on the dishware, sanitisation through reaching 3 600 HUE (Heat Unit Equivalents, according to NSF/ANSI 3). To ensure the right amount of rinse water, a rinse pump is used.

Unique features

· Eliminates any need for soaking, scrubbing or pre-washing 
· Low water consumption, average savings of 70 % compared to manual potwashing

· Low rinse volume, from 4 litres per wash cycle

· Low energy consumption, up to 60 % savings compared to manual potwashing

· Low detergent consumption, up to 60 % savings compared to manual potwashing

· Only 1 m2 space needed for the machine

· Capacity per hour (pots & pans): 96 GN 1/1 up to 200 mm depth
· Unique capacity per cycle: 4 GN 1/1 or 8 GN 1/2 (possible to load without extra accessories)

· Complying with Hygiene guidelines and norms DIN 10512 and NSF/ANSI 3,
· Ergonomic for the Operator, control panel at eye level 
· 3 programs with Granules and 3 highly efficient powerful water programs,
· Count-down of remaining program time shown in the display during the program

· Led lights alerting when wash cycle is finished,

· HACCP log accessed via a USB port or the Display
· Possible to add heat exchanger as an option
Construction material

· Wash cabin including wash tank are made of high-quality stainless steel, EN 1.4301. High level of recyclable materials is used in all parts of the product.
Hygiene / HACCP
Food-safety is secured through a controlled amount of high temperature rinse water, fulfilling hygiene requirements. 
· The rinse with 8 l of fresh water of 85 °C for 40 seconds, ensures a surface temperature of min 72 ºC on the wash ware. Disinfection conforms to NSF/ANSI 3 (reaching 3600 HUE). 
· The rinse with 4 l of fresh water of 85 °C for 20 seconds (ECO Programme), ensures a high surface temperature on the dishware. Disinfection conforms to DIN 10512 standard. 
Two integrated systems for hygiene/HACCP and operational data information


1) The latest 200 wash cycles can be viewed via the display.
2) Possible to download log data via USB port, the latest 200 wash cycles are available.
Available data:
· Date/Hour 
· Type of programme / ECO/Short/Normal 
· Washing with Granules is marked with an additional ”G”
· Average temperature in the wash tank during the cycle is marked with e.g. „65“
· Average temperature in the rinse tank during rinsing is marked with e.g. „85“
· “OK“ indicates correct amount of rinse water volume
· Error codes
· Accumulated values for water and energy consumption


Built-in intelligence – GD Memo™

Indications and reminders for the operator when it is 1) time to exchange PowerGranules, 2) renew tank wash water or 3) time for periodical service.

Detergent and rinse agent
For a good and hygienic wash result the machine needs detergent and rinse aid. 230 VAC signals available in separate connection box for detergent, rinse agent, dosing equipment and probes. 
PowerGranules®
· The machine operates with 6 liters of long-lasting PowerGranules®, made of POM (PolyOxyMethylene copolymer). PowerGranules® are especially developed for GRANULDISK machines.
· One load of long-lasting PowerGranules® are recirculated in the potwasher for in average 2500 wash cycles and can wash as many as up to 15.000 GN 1/1 or equivalent in other pots and pans.
· The material in the PowerGranules® is approved from FDA to be used in contact with food.

PowerGranules BIO®
· As a green alternative to original PowerGranules®, the potwasher can operate with 8 litres of biodegradable PowerGranules BIO® 

· PowerGranules BIO® are made from biodegradable esters, calcium carbonate and rapeseed oil.

· PowerGranules BIO® are specially developed for use in GRANULDISK machines.
· The material in the PowerGranules BIO® is approved to be used in contact with food.

· One load of long-lasting Granules is recirculated in the potwasher for in average 2500 wash cycles and can wash as many as up to 15.000 GN 1/1 or equivalent in other pots and pans.
TECHNICAL DATA GRANULE FLEXI

GENERAL.

	Design
	Front loaded with two level washing

	Outer dimensions (H x W x D)
	1,770 mm (±25) x 875 mm x 965 mm

	Loading volume
	205 litres (H 650 x W 600 x D 530 mm)

	Loading dimensions
	Pots up to the maximum rack dimensions; Ø 500 mm with 650 mm depth – or equivalent in other utensils.

	Weight (filled / empty)
	369 kg / 275 kg

	Wash tank volume
	84 litres

	Rinse water volume
	4 litres (ECO programme) complies with DIN 10512 
8 litres (Short / Normal programme) complies with NSF/ANSI-3

	Granule volume
	6 litres

	Wash water temperature
	65 °C

	Rinse water temperature
	85 °C

	Sound pressure level
	<70 dB(A) (operating with granules)


PROGRAMMES & CAPACITY.

	Wash programmes with Granules
	ECO: 2 min, 30 sec  
Short: 4 min, 40 sec
Normal: 6 min, 40 sec

	Wash programmes without Granules
	ECO: 2 min, 30 sec
Short: 3 min, 40 sec
Normal: 5 min, 40 sec

	Steam reduction 
	Adds 60 sec to Short/Normal programs. No steam reduction on Eco cycle. Feature can be turned off.

	EcoExchanger
	Adds 70 sec to Eco programs. Adds 80 sec to Short/Normal programs. Feature can be turned off.

	Max. / normal capacity per hour (normal capacity incl. handling)
	96 / 84 GN 1/1 or similar in other pots 

	Capacity per programme
	4 GN 1/1 or 8 GN 1/2 or 1x GN 2/1 + 2x GN 1/1, max. 200 mm depth, or similar in other pots.


ELECTRICAL CONNECTION.

	Voltage**
	3~ 400/415 V/50 Hz + PE  
3~ 230 V/50 Hz + PE 
3~ 380 V/50 Hz + PE

	Fuses (at max output 12.0 kW)
	25 A (400/415 V, 380 V) / 40 A (230 V)

	Fuses (at max output 17.0 kW)
	32 A (400/415 V, 380 V)

	Max output, hot fill
	12.0 kW

	Max output, cold fill
	17.0 kW

	Wash pump motor
	2.2 kW

	Rinse pump motor
	0.37 kW

	Heater wash tank*
	9 kW at output 12.0 kW (14 kW at output 17.0 kW)

	Heater rinse tank*
	9 kW at output 12.0 kW (14 kW at output 17.0 kW)

	Enclosure rating
	IPX5


WATER CONNECTION.

	Hardness / Connection
	Total hardness: 3-7 °dH, 5-12 °TH, 53-125 PPM / 
¾” BSP male (DN20). Note: reversed osmosis or distilled water must not be used.

	Connection A
	12.0 kW unit: 1-6 bar, 15 l/min, 55-65 °C (Hot fill)
17.0 kW unit: 1-6 bar, 15 l/min, 5-65 °C (Cold fill, optional)

	Connection B
	12.0/17.0 kW unit: 3-6 bar, 15 l/min, <20 °C

	Back flow protection
	EN 1717 / return flow protection of type AB with an air gap in accordance with EN 13077.


DRAIN & VENTILATION.

	Ventilation recommendation with steam reduction
	150 m3/h

	Ventilation recommendation without steam reduction
	400 m3/h

	Ventilation recommendation with EcoExchanger
	75 m³/h

	Heat emitted to room with steam reduction (total/sensible/latent)
	1.2 kW / 0.5 kW / 0.7 kW

	Heat emitted to room without steam reduction (total/sensible/latent)
	1.7 kW / 0.5 kW / 1.2 kW

	Heat emitted to room with EcoExchanger (total/sensible/latent)
	0.9 kW / 0.5 kW / 0.4 kW

	Drainage pipe
	Pipe Ø 32 mm

	Drain
	Capacity requirement 50 l/min. No drain pump, gravity drain. 
The machine must not be placed directly over a drain.

	Detergent and rinse aid
	Required. 230 VAC signal is available from a separate box. Max load total 0.5 A


 * Never operate simultaneously.
** For all available voltage options, please turn to your sales representative or consult a valid pricelist.

Granule Flexi® is supplied with a 2-meter-long power cable (without plug) for connection to an approved mains switch. The mains switch is not included in the delivery.
PAGE  
1
EN_Tender-text_Granule Flexi_32A_cold feed_2020.doc

