Standard delivery:

- Compact and versatile cookware, dishware and glassware washing machine - Automatic hood lift operated by the hands-free foot mechanism or by push on the operating panel - Safety crush protection and detection of obstacles - Manual override for the hood by the stainless-steel ergonomic handle if electrical breakdown.

- Can be installed along a wall or at a corner - Operating panel positioned at eye level and installable on site on right or left side.

- Stainless steel construction - Rear cover plate - Hood double skinned - Noise & heat insulated hood.

- Liquid detergent and rinse aid dosing dispenser with peristaltic pump - Suction hoses - Prepared to detergent dosing sensor installation.

- Drain and discharge pump with water drain lifting height of 800 mm.

- Basket loading size: 500 x 500 mm - Removable rack holder - Clear passing height: 430 mm - Working height: 860 - 910 mm.

- QR code with immediate online access to operator training videos - servicing history - troubleshooting guides.

- Wifi connectivity preparation: HACCP Data collection and history - Control of washing parameters - Tele service with detection of defective component - Apps to load on Google Play or Apps Store.

- Starter kit: 1 Stainless steel cookware wash basket - 1 Polypropylene dishware wash basket - 1 Polypropylene glassware wash basket.

- Installation kit: Supply hose 3/4" female - Drain hose Ø 28 mm - 2 m. electrical cable.

Efficient washing and rinsing technology:

- Automatic activation of wash mode by wash basket detection sensor.

- Cookware washing technology: 2 Stainless steel oscillating arms driven by motor and removable without tool and positioned below the wash basket - Stainless steel nozzles - 1 High pressure wash pump 1,5 kW.

- Dishware and Glassware washing technology: 2 Stainless steel rotating arms driven by water pressure and removable without tool - 1 over the wash basket, 1 below - Printed nozzles - 1 Low pressure wash pump 1,1 kW.

- Rinsing technology: 2 Stainless steel arms removable without tool and with optimized pressure system - 1 over the washing basket, 1 below - Stainless steel nozzles - 1 rinse pump 0,18 kW.

- Stamped wash tank without any welding: 30 L - 2 stage filter system - Heating element 8 kW.

- Rinse tank: 12 L - Heating element 8 kW.

User benefits:

- Easy to use operating panel with washing program icons, count down, wash and rinse tank temperature.

- Intuitive operating panel with waterproof colored TFT screen display within easy reach.

- Total starting time (Fill and heat water): 9 min. with 55°C water inlet.

- Water change time (Drain, fill and heat): 5 min. with 55°C water inlet.

- Cookware wash mode - 2 programs: 3 min.(cookware) - 6 min. (carbonized cookware) .

- Dishware and Glassware wash mode - 2 programs: 2 min. (plates, cutlery, utensils) - 1 min. (glasses).

- Self-cleaning program.

- Auto start wash cycle.

- Wash temperature: Factory setting 60°C - Adjustable from 35°C up to 65°C for each wash program independently.

- Rinse temperature: Factory setting 85°C - Adjustable from 65°C up to 85°C for each wash program independently.

- Programmable shutdown timer.

Capacity per wash cycle:

- Cookware wash mode: Pot up to Ø 450 H 380 mm or sub sized - 1 GN 1/1 Depth 200 mm or sub-sized - 2 GN 1/1 Depth 65 mm.

- Dishware and Glassware wash mode: 18 Dinner plates or 12 Soup plates Ø 240 mm - 25 Glasses/mugs ∅ 90 mm.

Consumptions per wash cycle:

- Water: 2.6 L – Energy 0.176 kWh.

- Water and energy consumption management - Energy saving mode if not used for 20 min.

Dimensions - Connections - Standards and certification compliance:

- Dimension: H 1950 mm x W (655+100 handle) x D (785+95 handle) - Side operating panel L 430 mm.

- Weight: Net 150 Kg, Gross 190 Kg.

- Water inlet: 55°C/60°C - 2-5 bar - < 4 °Dh (7 °f) - 70 PPM - 3/4" female. - Water outlet: Ø 28 mm - 65°C - ⩾ 50 L/min.

- Connected load: 16 A - 10 kW - 3N AC 380-400-415 V - 50 Hz.

- < 64 dB(A) - IPX5 - CE - DIN 10534 - EN 1717 - RoHS - WEEE.

Factory fitted options:

- Other voltage and frequency: 27 A - 10 kW - 3N AC 230 V - 50 Hz.

- Inbuilt exhaust heat recovery with steam condensing unit - Self-cleaning function - 10°C/25°C water supply - Wash cycle extended of 30 sec. - Dimension of the machine equipped of the option: H 2125 mm x W (655+100 handle) x D (785+95 handle).

Water treatment option:

- Non-electric external water softener: 2°C /65°C Water supply - 8 kg Salt capacity - Regeneration cycle at 2 bar: water 34 L, time 11 min Dimension: H 500 x L 200 mm x D 450 mm - Weight: Net 21 Kg, Gross 39 Kg - Water connection: Inlet 3/4" male, Outlet 3/4" male, Drain 1/2".

High productivity options:

- Pots and pans scrubber with ergonomic handle - Thermal resistant to 100°C - Dimension: L 101 x D 152 x H 76 mm - Packing 12 units.

- Stainless steel cookware wash basket - L 500 x D 500 x H 85 mm - Capacity: Pot up to Ø 450 H 380 mm or sub sized - 1 GN 1/1 Depth 200 mm or sub-sized - 2 GN 1/1 Depth 65 mm.

- Polypropylene dishware wash basket - L 500 x D 500 x H 110 mm - Capacity: 18 Dinner plates or 12 Soup plates Ø 240 mm.

- Polypropylene glassware wash basket - L 500 x D 500 x H 118 mm - Capacity: 25 Glasses or mugs ∅ 90 mm.

- Polypropylene cutlery wash holder - L 105 x D 105 x H 140 mm - Fits with polypropylene dishware wash basket.